Prvo skinem virtualnu masinu sa ucilnice(desni klik save as i izaberem particiju dje moze), skinem je u neki folder i ekstrakujem. Udjem u VM Box i idem na +(open) nadjem folder dje sam ekstrakovao i onda kliknem na plavu .vbox file. Startujem masinu i odem u Devices->shared clipboard-> bidirectional da omogucim copy paste.

Vazda uzeti NAT network, odem u Network pa u NAT i dodam na + novu. Obavezno refresh mac adresu. Kada kliknem desni klik na image dodam u network NAT network i izaberem kreirani.

Prvo da ugasim IPv6 jer smeta, udjem sudo nano /etc/sysctl.conf i unesem ove komande:

net.ipv6.conf.all.disable_ipv6 = 1

net.ipv6.conf.default.disable_ipv6 = 1

net.ipv6.conf.lo.disable_ipv6 = 1

Da bih aktivirao promjene unesem sudo sysctl –p, da provjerim je li dobro unijeto unosim komandu cat /proc/sys/net/ipv6/conf/all/disable_ipv6, dobicu 1 kao rezultat ako je dobro.

Da vidimo ip adresu kucam komandu ip addr.

Za skidanje fajlova koristim komandu sudo apt install <ime fajla>, a za azuriranje sudo apt update.

Da ugasim masinu koristim sudo poweroff.

Kad ocu da kloniram fajl kucam git clone #link

Da aktiviram firewall rules sudo ./iptables1.sh start

Da resetujem na default:  sudo ./iptables1.sh reset

Da vidim aktivne rules: sudo iptables --list –vn

Kada imam neke podmreze, na ruteru stavim Adapter1 NAT, a npr ako imam jos 2 podmreze aktiviram Adapter2 i Adapter3 i stavim ih na Internal Network i dam im odgovarajuca imena. Onda udjem na ruteru u sudo nano /etc/netplan/01-network-manager-all.yaml i stavim vrijednosti za enp0s8 itd respektivno kako je zadato npr addresses: [10.0.0.1/24]. Da bih sacuvao te postavke pokrenem sudo netplan apply.

Omoguciti rutiranje za IPv4, da ruter moze da se ponasa kao ruter sa komandom: echo 1 | sudo tee /proc/sys/net/ipv4/ip_forward.

Da omogucim rutiranje iz private mreze ka internet pokrenem sudo iptables -t nat -A POSTROUTING -o enp0s3 -j MASQUERADE.

Kada trebam da podesim ip adresu itd u racunaru u podmrezi udjem u: sudo nano /etc/netplan/01-network-manager-all.yaml. I dopunim ovako, samo stavim podatke koji mi tacno trebaju:
network:

version: 2

ethernets:

enp0s3:

# assign the IP address

addresses: [10.0.0.2/24]

# set the default route through isp

routes:

- to: default

via: 10.0.0.1

# use Google's DNS

nameservers:

addresses: [8.8.8.8]

SSH PROTOCOL

Odradim sudo apt update da azuriram pakete, a da instaliram nove kucam sudo apt install <list-the-packages>, na ucilnici je spisak komandi

Uzmem kloniram tu masinu i dobijem 2 masine, ugasim sve adaptere osim prvog koji je na NAT network.

SSH SERVER:

Otvorim fajl /etc/hosts i dodam ovu liniju 127.0.1.1 ssh-server, sacuvam fajl i odradim komandu sudo hostnamectl set-hostname ssh-server. Restartujem terminal, i sada sam promijenio hostname masine.

Sad regenerate SSH server keys, prazne passphrase ostavljam.

sudo ssh-keygen -t ecdsa -f /etc/ssh/ssh_host_ecdsa_key

sudo ssh-keygen -t rsa   -f /etc/ssh/ssh_host_rsa_key

sudo ssh-keygen -t dsa   -f /etc/ssh/ssh_host_dsa_key

sudo ssh-keygen -t ed25519 -f /etc/ssh/ssh_host_ed25519_key

Na SSH CLIENT udjem i ukucam isp@IpAdresaServera, pritisnem yes i ukucam password. Logout radim sa exit ili logout ili ctrl+d.

Za ostatak uci na ucilnicu pa ici korak po korak, ne vjerujem da ce doci na kolokvijumu.

VPN IPsec

Znaci imam dvije privatne mreze koje treba spojiti sa IPsec, znaci imam 2 rutera i dva racunara. Uzmem base image i instaliram sve potrebno

sudo apt install \

charon-systemd \

strongswan-swanctl \

strongswan-pki \

libstrongswan-extra-plugins \

apache2 \

wireshark

Prvo provjeriti je li strongswan upaljen sa sudo systemctl status strongswan ako pise running onda je ok, ako ne pise unesem komandu sudo systemctl enable --now strongswan

Ugasim masinu i podesim Adapter1 na NAT Network, a Adapter2 na Internal network i stavim bilo sta. Sad to kloniram 4 puta, i onda tu podesavam na Adapter2 imena. Na ruterima kreiram te subnet u Internal Network, a na racunarima izaberem te odgovarajuce. Onda udjem da konfigurisem ruter, prvo sam uzeo hq_ruter. Otvorim /etc/netplan/01-network-manager-all.yaml i konfigurisem po zadatku, dodam adresu podmreze, npr:

network:

version: 2

ethernets:

enp0s3:

dhcp4: true

dhcp-identifier: mac

enp0s8:

addresses: [10.1.0.1/16] #adresa podmreze

Da aktiviram promjene  sudo netplan apply, i za ruter ovo vazda da pokrenem da ne zaboravim !!!! echo 1 | sudo tee /proc/sys/net/ipv4/ip_forward da bi mogao rutirati

Sad podesavam racunar u ovoj mrezi, dajem mu staticku konfiguraciju kao sto je trazeno, otvorim /etc/netplan/01-network-manager-all.yaml
network:

version: 2

ethernets:

enp0s3:

addresses: [10.1.0.2/16] #adresa staticna

routes:

- to: default

via: 10.1.0.1

nameservers:

addresses: [8.8.8.8]

Da aktiviram promjene  sudo netplan apply

Sad predjem na drugi ruter i sve isto uradim:
network:

version: 2

ethernets:

enp0s3:

dhcp4: true

dhcp-identifier: mac

enp0s8:

addresses: [10.2.0.1/16] #adresa podmreze

Da aktiviram promjene  sudo netplan apply, i za ruter ovo vazda da pokrenem da ne zaboravim !!!! echo 1 | sudo tee /proc/sys/net/ipv4/ip_forward da bi mogao rutirati

I za racunar u ovoj podmrezi, otvorim /etc/netplan/01-network-manager-all.yaml

network:

version: 2

ethernets:

enp0s3:

addresses: [10.2.0.2/16]

routes:

- to: default

via: 10.2.0.1

nameservers:

addresses: [8.8.8.8]
Da aktiviram promjene  sudo netplan apply

Da bi kreirao IPsec tunel na jednom ruteru npr hq_ruter otvorim /etc/swanctl/swanctl.conf i na vrhu dodam:
connections {

net-net {

# Use IKEv2

version = 2

# Remote "public" address: the IP of branch_router on enp0s3

# Replace $BRANCH_IP with the actual address you see on branch_router.

remote_addrs = $BRANCH_IP

# IKE authentication: pre-shared key (PSK)

local {

auth = psk

id   = hq

}

remote {

auth = psk

id   = branch

}

children {

net-net {

# Local HQ subnet behind hq_router

local_ts  = 10.1.0.0/16

# Remote branch subnet behind branch_router

remote_ts = 10.2.0.0/16

# We will initiate the tunnel manually via swanctl,

# so we keep the default start_action = none.

# (You could also use "trap" to start on demand.)

start_action = none

}

}

}

}

secrets {

# IKE pre-shared key shared between @hq and @branch

ike-hq-branch {

id-hq     = hq

id-branch = branch

secret    = "secret"

}

}

Sad reloadujem sa sudo swanctl --load-all

Sada na drugom ruteru otvorim /etc/swanctl/swanctl.conf, i dodam:
connections {

net-net {

version = 2

# Remote "public" address: the IP of hq_router on enp0s3

# Replace $HQ_IP with the actual address you see on hq_router.

remote_addrs = $HQ_IP

local {

auth = psk

id   = branch

}

remote {

auth = psk

id   = hq

}

children {

net-net {

# Local branch subnet behind branch_router

local_ts  = 10.2.0.0/16

# Remote HQ subnet behind hq_router

remote_ts = 10.1.0.0/16

start_action = none

}

}

}

}

secrets {

ike-hq-branch {

id-hq     = hq

id-branch = branch

secret    = "secret"

}

Sad reloadujem sa sudo swanctl --load-all, da provjerim ipsec podesavanja ukucam sudo swanctl --list-conns

Da pokrenem tunel kucam sudo swanctl --initiate --child net-net a da ga ugasim sudo swanctl --terminate --ike net-net.

RADIUS AAA

Prvo instaliram sve sto treba, sudo apt install freeradius freeradius-utils apache2 libapache2-mod-auth-radius wireshark

Verzija dje se pravi server i pokazuje da radi, otvorim sudo nano /etc/freeradius/3.0/clients.conf i unosim da registrujem NAS

client localhost {

ipaddr = 127.0.0.1

secret = testing123

require_message_authenticator = no

nas_type = other

}

Da dodam end usera otvaram sudo nano /etc/freeradius/3.0/users, i dodam tekst

"alice" Cleartext-Password := "password"

Prvo ugasim server sa sudo service freeradius stop otvorim novi terminal i pokrenem sudo freeradius -X -d /etc/freeradius/3.0

Da vidim radi li kucam echo "User-Name=alice, User-Password=password" | radclient 127.0.0.1 auth testing123 –x

PRIMJER SA WEB SERVEROM I RADIUS NA UCILNICI

ROAD WARRIOR IPSEC

Na road warrioru u swanctl.conf stavim novu konekciju:

connections {

rw {

version = 2

remote_addrs = #adresa rutera

vips = 0.0.0.0

local {

auth = psk

id = “adresa roadwarriora” #ako je zadato neko ime stavim ime npr alice, ako dobijem psk koji treba stavim #”sifra” dolje u connections

}

remote {

auth = psk

id = “adresa rutera” #ili ime

}

children {

rw {

remote_ts = “subnet rutera”

start_action = none

}

}

}

}

secrets {

ike-rw {

secret = “secret”

}

}

A na ruteru dodam konekciju u swanctlconf:

connections {

rw {

version = 2

local_addrs  = #adresa rutera

pools = rw_pool

local {

auth = psk

id = #adresa rutera

}

remote {

auth = psk

id = ime #ako imam zadato ime, a u secrets dolje kucam psk koji je zadat

}

children {

rw {

local_ts  = #lokalni subnet

}

}

}

}

secrets {

ike-rw {

secret = “secret”

}

}

pools {

rw_pool {

addrs = 10.3.0.0/28

}

}

----------------------------------------------------

zabrani input output saobracaj, samo forward dozvoljen
iptables -P INPUT DROP

iptables -P OUTPUT DROP

iptables -P FORWARD ACCEPT

Omoguci vec postavljene konekcije
iptables -A INPUT  -m state --state ESTABLISHED,RELATED -j ACCEPT

iptables -A OUTPUT -m state --state ESTABLISHED,RELATED -j ACCEPT

LOOP BACK OBAVEZNO

iptables -A INPUT  -i lo -j ACCEPT

iptables -A OUTPUT -o lo -j ACCEPT

Incoming pravila sa internet:

ICMP - iptables -A INPUT -i enp0s3 -p icmp -m state --state NEW -j ACCEPT

ISAKMP - iptables -A INPUT -i enp0s3 -p udp --dport 500 -m state --state NEW -j ACCEPT

NAT-T- iptables -A INPUT -i enp0s3 -p udp --dport 4500 -m state --state NEW -j ACCEPT

ESP - iptables -A INPUT -i enp0s3 -p esp -m state --state NEW -j ACCEPT

Izlazni saobracaj:

ICMP - iptables -A OUTPUT -o enp0s3 -p icmp -m state --state NEW -j ACCEPT

DNS - iptables -A OUTPUT -o enp0s3 -p udp --dport 53 -m state --state NEW -j ACCEPT

Kada imam freeradius i treba dodati neki NAS, otvaram sudo nano /etc/freeradius/3.0/clients.conf i unosim

client nas-gateway {

ipaddr = #ipaddr

secret = #password

}

Korisnika dodajem tako sto otvorim fajl sudo nano /etc/freeradius/3.0/users

it u unesem alice(username bez navodnika) Cleartext-Password := "alice"(password)

FORWARD PRAVILA
iptables -A FORWARD -i enp0s8 -o enp0s3 -j ACCEPT

iptables -A FORWARD -i enp0s3 -o enp0s8 -m state --state ESTABLISHED,RELATED -j ACCEPT